# Pratical1:

Design and develop a reprogrammable embedded computer using 8051 microcontrollers and to show the following aspects.

- Programming

- Execution

- Debugging Code:

#include<reg52.H>

sbit LED = P2^0;

void Delay(void);

void main(void)

{

while(1)

{

LED = 0;

Delay();

LED = 1;

Delay();

}

}

void Delay(void)

{

int j;

int i;

for(i = 0; i < 10; i++)

{

for(j = 0; j < 10000; j++)

{

}

}

}

Output:

| 2. A | Configure timer control registers of 8051 and develop a program to generate given time delay. |
| --- | --- |

Code:

#include<reg51.H>

void Delay(void);

void main(void)

{

while(1)

{

P1 = 0xFF; // Make all bits of P1 high

Delay();

P1 = 0x00; // Make all bits of P1 low

Delay();

}

}

void Delay(void)

{

int j;

int i;

for(i = 0; i < 1000; i++)

{

}

for(j = 0; j < 1000; j++)

{

}

}

Output:

| B | To demonstrate use of general purpose port i.e. Input/ output port of two controllers for data transfer between them. |
| --- | --- |

Code control 1:

#include<reg51.h>

void Delay(unsigned int time);

void main(void)

{

P2 = 0xAA;

Delay(1000);

P2 = 0x55;

Delay(1000);

}

void Delay(unsigned int time)

{

unsigned int i,j;

for (i = 0; i < time; i++){

for (j = 0; j < 23; j++){

}

}

}

Control2 :

#include<reg51.h>

void Delay(unsigned int time);

void main(void)

{

while(1){

P2 = P1;

Delay(1000);

}

}

void Delay(unsigned int time)

{

unsigned int i,j;

for (i = 0; i < time; i++){

for (j = 0; j < 23; j++){

}

}

}

Output:

| 3. A | Port I / O: Use one of the four ports of 8051 for O/P interfaced to eight LED’s. Simulate binary counter (8 bit) on LED’s |
| --- | --- |

Code:

#include<reg51.h>

void delay(int time);

void main()

{

P1 = 00000000;

while(1)

{

P1++;

delay(100);

}

}

void delay(int time)

{

int i, j;

for(i = 0; i <= time; i++)

{

for(j = 0; j <= 23; j++)

{

}

}

}

Output:

| B | To interface 8 LEDs at Input-output port and create different patterns. |
| --- | --- |

3b} Code:

#include<reg51.h>

void delay();

void main()

{

while(1)

{

P1 = 0xAA; // Pattern to turn on alternate LEDs - 10101010

delay();

P1 = 0x55; // Reverse the pattern

delay();

}

}

void delay()

{

unsigned int i, j;

for(i = 0; i < 23; i++)

{

for(j = 0; j < 1000; j++)

{

}

}

}

Output:

3c

| 3C | To demonstrate timer working in timer mode and blink LED without using any loop delay routine. |
| --- | --- |

Code:

#include<reg51.h>

int i = 0;

void timer_ISR(void)interrupt 1

{

i++;

if(i == 10)

{

i=0;

P1++;

}

}

void main(void)

{

TMOD = 0x01;

ET0 = 1;

TR0 = 1;

EA = 1;

while(1);

} Output

| 4. A | Serial I / O: Configure 8051 serial port for asynchronous serial communication with serial port of PC exchange text messages to PC and display on PC screen. Signify end of message by carriage return. |
| --- | --- |

Code:

#include<reg51.h>

void send(char x);

void main(void)

{

TMOD = 0x20;

TH1 = 0xFD;

SCON = 0x50;

TR1 = 1;

send('M');

send('O');

send('H');

send('A');

send('M');

send('M');

send('A');

send('D');

while(1);

}

void send(char x)

{

SBUF = x;

while(TI == 0);

TI = 0;

}

Output:

| 4B | To demonstrate interfacing of seven-segment LED display and generate counting from 0 to 99 with fixed time delay. |
| --- | --- |

Code:

#include<reg51.h>

void delay(unsigned int ms)

{

unsigned int i, j;

for(i = 0; i<ms; i++)

{

for(j = 0; j<=1275; j++)

{

}

}

}

void main(void)

{

char number[] = {0x3F, 0x06, 0x5B, 0x4F, 0x66, 0x6D, 0x7D, 0x07, 0x7F, 0x6F};

int i, j;

P2 = 0x00;

P3 = 0x00;

while(1)

{

for(i = 0; i<=9; i++)

{

P2 = number[i];

for(j = 0; j<=9; j++)

{

P3 = number[j];

delay(50);

}

}

}

}

Output:

| 4C | Interface 8051 with D/A converter and generate square wave of given frequency on oscilloscope. |
| --- | --- |

Code:

#include<reg51.h>

void delay();

void main()

{

P2 = 0x00;

while(1)

{

P2 = 0xFF;

delay();

P2 = 0x00;

delay();

}

}

void delay()

{

int i;

for(i = 0; i <= 5000; i++)

{

}

}

Output:

| 5. A | Interface 8051 with D/A converter and generate triangular wave of given frequency on oscilloscope. |
| --- | --- |

Code:

#include<reg51.h>

void main()

{

P2= 0x00;

while(1)

{

do

{

P2 += 0x05;

}

while(P2 < 0xFF);

do

{

P2 -= 0x05;

}

while(P2 > 0x00);

}

}

Output:

| 5B] | Using D/A converter generate sine wave on oscilloscope with the help of lookup table stored in data area of 8051. |
| --- | --- |

Code:

#include<reg51.h>

#include<stdio.h>

int sine[]={0,0,5,10,5,15,20,25,30,35,40,45,45,40,35,30,25,20,15,10,5,0,0};

void main()

{

int i;

P1=0x00;

while(1)

{

for(i=0;i<23;i++)

{

P1=sine[i];

}

}

}

Output:

| 6. | Interface stepper motor with 8051 and write a program to move the motor through a given angle in clock wise or counter clock wise direction. |
| --- | --- |

Code:

#include<reg51.h>

void delay()

{

int i, j;

for(i = 0; i <= 100; i++)

{

for(j = 0; j < 100; j++)

{

}

}

}

void main()

{

while(1)

{

P2 = 0x09;

delay();

P2 = 0x03;

delay();

P2 = 0x06;

delay();

P2 = 0x0C;

delay();

}

}

Output:

| 7. | Generate traffic signal. |
| --- | --- |

Code:

#include<reg51.h>

sbit red = P2^0;

sbit yellow = P2^1;

sbit green = P2^2;

void delay(int time);

void main()

{

red = yellow = green = 0;

while(1)

{

red = 1;

delay(1000);

red = 0;

yellow = 1;

delay(200);

yellow = 0;

green = 1;

delay(1000);

green = 0;

yellow = 1;

delay(200);

yellow = 0;

}

}

void delay(int time)

{

int i, j;

for(i = 0; i < time; i++)

{

for(j = 0; j < 1000; j++)

{

}

}

}

Output: